Tremco, Inc. Commercial Sealants and Waterproofing

Section 07 14 16.05 COLD FLUID-APPLIED WATERPROOFING
Dual Waterproofing System

Specifier: This guide specification section specifies Tremco TREMproof® Platinum Waterproofing System. This system includes the use of either TREMproof 250GC or TREMproof 201/60 in multiple lifts with the TREMproof TRA Sheeting fully embedded.

• Designed for use on backfilled walls, split slab applications, planters
• When using TREMproof 250GC, it can be used on concrete in as few as 24 hours after removal of forms, dramatically expediting construction schedules.

Separate guide specification sections are available from Tremco for vertical, horizontal (deck), and vegetative roof applications. This section is easily edited using several common commercial specification software tools.

We recommend you consult with your Tremco construction technical representative, who can be contacted through: Tremco, Inc., Commercial Sealants and Waterproofing Division, Beachwood OH; (866) 321-6357); email: techresources@tremcoinc.com;  www.tremcosealants.com.

Tremco sealant and waterproofing products appear in the following CSI MasterFormat specifications sections:

• Section 07 01 91 Joint Sealant Rehabilitation and Replacement
• Section 07 14 13.01 Hot Fluid-Applied Waterproofing, Deck (TREMproof 6100)
• Section 07 14 13.02 Hot Fluid-Applied Waterproofing, Vegetated Roof (TREMproof 6100)
• Section 07 14 16.01 Cold Fluid-Applied Waterproofing, Vertical and Deck (TREMproof250GC)
• Section 07 14 16.02 Cold Fluid-Applied Waterproofing, Vertical (TREMproof250GC)
• Section 07 14 16.03 Cold Fluid-Applied Waterproofing, Deck(TREMproof 250GC)
• Section 07 14 16.04 Cold Fluid-Applied Waterproofing, Vegetative Roof (TREMproof250GC)
• Section 07 17 00 Bentonite Waterproofing (Paraseal GM/LG 60 mil)
• Section 07 18 00.01 Traffic Coatings, Vehicular
• Section 07 18 00.02 Traffic Coatings, Pedestrian
• Section 07 18 00.03 Traffic Coatings, Vehicular and Pedestrian
• Section 07 27 13 Modified Bituminous Sheet Waterproofing, Vapor-Retarding (ExoAir 110)
• Section 07 27 23 Board Product Waterproofing, Vapor Permeable (SecurerockExoAir 430)
• Section 07 27 26.01 Fluid-Applied Membrane Waterproofing, Vapor-Retarding (ExoAir 120)
• Section 07 27 26.02 Fluid-Applied Membrane Waterproofing, Vapor Permeable (ExoAir 220)
• Section 07 27 26.03 Fluid-Applied Membrane Waterproofing, Vapor Permeable (ExoAir 230)
• Section 07 84 13 Penetration Firestopping
• Section 07 84 46 Fire-Resistive Joint Systems
• Section 07 92 00 Joint Sealants
• Section 08 85 00 Glazing Sealants
• Section 32 13 73 Concrete Paving Joint Sealants

TREMproof® and TREMDrain® are registered trademarks of Tremco, Inc.
This document is Copyright© 2014 by Tremco, Inc.

SECTION 07 14 16 – COLD FLUID-APPLIED WATERPROOFING

- GENERAL

      - SUMMARY

        - Section Includes:

          - Cold fluid-applied waterproofing, dual waterproofing system for vertical and horizontal applications.

          - Waterproofing membrane, primer, protective waterproofing courese, and drainage.

      - RELATED REQUIREMENTS

Specifier: If retaining this optional Related Requirements Article, edit to include only those sections included in project manual.

          - Section 03 31 00 "Cast-in-Place Concrete" for moisture curing of concrete waterproofing substrate.

          - Section 04 20 00 "Unit Masonry" for compatibility with flashing components.

          - Division 07 air barrier section for wall waterproofing and interface coordination.

          - Section 07 72 73 "Membrane Leak Detection System" for requirements for EFVM leak detection system installation and membrane leak testing.

          - Section 07 76 13 "Roof Pavers" for requirements for roof ballast and roof decking pavers and support pedestal systems.

          - Section 07 92 00 "Joint Sealants" for joint sealants and accessories and joint preparation.

          - Section 07 95 00 "Expansion Control" for expansion joint systems.

          - Section 32 97 00 "Vegetated Roof Assemblies" for growing media, plantings, filter fabrics, root barriers, and related components.

          - Section 33 46 00 "Subdrainage" for drainage pipe and conduits, drainage panels, and filter fabrics.

      - REFERENCES

Specifier: If retaining this optional References Article, edit to include only those references cited in the edited section.

        - References, General: Versions of the following standards current as of the date of issue of the project apply to the Work of this Section.

        - ASTM International (ASTM): www.astm.org:

          - ASTM C 836 – Standard Specification for High-Solids Content, Cold Liquid-Applied Elastomeric Waterproofing Membrane for Use with Separate Wearing Course

          - ASTM C 920 - Standard Specification for Elastomeric Joint Sealants

          - ASTM D 4258 - Standard Practice for Surface Cleaning Concrete for Coating

          - ASTM D 4263 - Standard Test Method for Indicating Moisture in Concrete by the Plastic Sheet Method

          - ASTM D 4716 - Standard Test Method for Determining the (In-plane) Flow Rate per Unit Width and Hydraulic Transmissivity of a Geosynthetic Using a Constant Head

          - ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials

          - ASTM E 96/E 96M - Standard Test Methods for Water Vapor Transmission of Materials

        - UL Environment Greenguard Certification: www.greenguard.org

          - Greenguard Certification Product Guide

        - U. S. Environmental Protection Agency (EPA): www.epa.gov:

          - 40 CFR 59, Subpart D - National Volatile Organic Compound Emission Standards for Architectural Coatings

      - ADMINISTRATIVE REQUIREMENTS

        - Preinstallation Conference: Conduct conference at Project Site.

          - Review requirements for waterproofing products and installation, including surface preparation, substrate conditions, project and manufacturer's details, installation procedures, mockups, testing and inspection requirements, protection and repairs, and coordination and sequencing of waterproofing work with work of other Sections.

      - ACTION SUBMITTALS

        - Product Data: For each type of waterproofing product specified, including:

          - Technical data indicating compliance with requirements.

          - Substrate preparation instructions and recommendations.

        - Shop Drawings: Show locations for waterproofing system components. Show details for each type of substrate, joints, corners, and edge conditions, including flashings, counterflashings, penetrations, transitions, and terminations.

      - INFORMATIONAL SUBMITTALS

        - Qualification Data: For Installer, manufacturer[, and waterproofing Inspector].

          - Certification of manufacturer's approval of Installer.

        - Low-Emitting Product Certificate: For waterproofing products specified to meet volatile organic emissions standards, submit Greenguard Children and Schools Certification or comparable certification acceptable to Architect.

        - Product Test Reports: Test data for waterproofing products and waterproofing system, by qualified testing agency, indicating proposed waterproofing meets performance requirements, when requested by Architect.

        - Warranty: Sample of unexecuted manufacturer and installer special warranties.

        - Field quality control reports.

      - QUALITY ASSURANCE

        - Installer Qualifications: A manufacturer-approved firm with minimum [three] years experience in installation of specified products in successful use on similar projects, employing workers trained by manufacturer, including a full-time on-site supervisor with a minimum of [three] years experience installing similar work, and able to communicate verbally with Contractor[, Architect,] and employees.

        - Manufacturer Qualifications: A qualified manufacturer [listed in this Section] with minimum five years experience in manufacture of waterproofing as one of its principal products.

          - Manufacturer's product submitted has been in satisfactory operation on five similar installations for at least five years.

Specifier: Retain "Approval of Manufacturers and Comparable Products" Paragraph below to provide control over qualifying of substituted manufacturers.

          - Approval of Manufacturers and Comparable Products: [Submit] [Prime bidder must submit] the following in accordance with project substitution requirements, within time allowed for substitution review:

            - Completed and signed Substitution Request form.

            - Product data, including certified independent test data indicating compliance with requirements.

            - Sample shop drawings from similar project.

            - Project references: Minimum of five installations of similar system not less than five years old, with Owner and Architect contact information.

            - Name and resume of proposed qualified Inspector.

            - Sample warranty.

Specifier: Retain "Waterproofing Inspector Qualifications" Paragraph if Contractor is required to provide manufacturer inspections under Part 3 Field Quality Control article.

        - Waterproofing Inspector Qualifications: A technical representative of manufacturer not engaged in the sale of products and experienced in the installation and maintenance of the specified waterproofing system, qualified to perform observation and inspection specified in Field Quality Control Article, to determine Installer’s compliance with the requirements of this Project, and approved by the manufacturer to issue warranty certification. The Inspector shall be one of the following:

          - An authorized full-time technical employee of the manufacturer.

          - An independent party certified as a waterproofing inspector acceptable to Architect, retained by the Contractor.

Specifier: Retain "Testing Agency Qualifications" Paragraph if Contractor is required to provide independent inspections under Part 3 Field Quality Control article.

        - Testing Agency Qualifications: Qualified independent agency experienced in the installation of the specified waterproofing system, and qualified to perform observation and inspection specified in Field Quality Control Article to determine Installer’s compliance with the requirements of this Project, acceptable to Architect, retained by the Contractor.

        - Mockups: Provide waterproofing mockup application within mockups required in other sections, or if not specified, in an area of not less than 150 sq. ft. (14 sq. m) of surface where directed by Architect for each type of substrate condition. Include examples of surface preparation, crack and joint treatment, waterproofing application, and flashing, transition, and termination conditions, to set quality standards for execution.

          - Include intersection of deck waterproofing with adjacent vertical waterproofing and moisture control systems, including Air Barrier Membrane(s)

      - DELIVERY, STORAGE AND HANDLING

        - Accept materials on site in manufacturer's unopened original packaging.

        - Store products in weather protected environment, clear of ground and moisture, within temperature ranges recommended by waterproofing manufacturer.

Specifier: Retain first option in "Construction Waste" Paragraph below for LEED projects; retain second option for other projects.

        - Construction Waste: Store and dispose of packaging materials and construction waste in accordance with requirements of Division 01 Section ["Construction Waste Management"] ["Temporary Facilities and Controls."]

      - ENVIRONMENTAL REQUIREMENTS

        - Environmental Limitations: Apply waterproofing within the range of ambient and substrate temperatures recommended by air-barrier manufacturer.

          - Protect substrates from environmental conditions that affect air-barrier performance.

          - Do not apply waterproofing to a damp or wet substrate or during snow, rain, fog, or mist.

      - SCHEDULING

        - Coordinate installation of waterproofing with completion of air barrier installation, coatings, roofing and other work requiring interface with waterproofing.

        - Schedule work so waterproofing applications may be inspected prior to concealment.

        - Ensure waterproofing materials are cured before covering with other materials.

      - WARRANTY

Specifier: Consult Tremco representative for available special project warranty terms and conditions.

        - Special Manufacturer's Warranty: Manufacturer's standard form in which waterproofing manufacturer agrees to furnish and install waterproofing material to repair or replace those materials installed according to manufacturer's written instructions that exhibit material defects or otherwise fail to perform as specified under normal use within warranty period specified.

          - Access for Repair: Owner shall provide unimpeded access to the Project and the waterproofing system for purposes of testing, leak investigation, and repair, and shall reinstall removed cladding and overburden materials upon completion of repair.

          - Cost Limitation: Manufacturer's obligation for repair or replacement shall be limited to the original installed cost of the work.

          - Warranty Period: [__] years date of Substantial Completion.

        - Special warranties specified in this article exclude deterioration or failure of waterproofing materials from the following:

          - Movement of the structure caused by structural settlement or stresses on the waterproofing exceeding manufacturer's written specifications for elongation.

          - Mechanical damage caused by outside agents.

- PRODUCTS

      - MANUFACTURERS

Specifier: Retain option for substitutions below and edit if required for project.

        - Basis-of-Design Products: Provide waterproofing products manufactured by Tremco, Inc., Commercial Sealants and Waterproofing Division, An RPM Company, Beachwood OH; (866) 321-6357; email: techresources@tremcoinc.com; www.tremcosealants.com, [or comparable products of other manufacturer approved by Architect in accordance with Instructions to Bidders and Division 01 General Requirements].

        - Source Limitations: Provide waterproofing system materials and accessory products from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

        - General: Waterproofing system shall be capable of performing as a continuous watertight installation and as a moisture drainage plane transitioned to adjacent flashings and discharging water to the building exterior. Waterproofing shall accommodate normal substrate movement and seal expansion and control joints, construction material transitions, opening transitions, penetrations, and perimeter conditions without resultant moisture deterioration.

Specifier: Paragraph below may apply to limitations on VOCs of authorities having jurisdiction; verify local requirements.

        - VOC Content: 250 g/L maximum per 40 CFR 59, Subpart D (EPA Method 24) and complying with requirements of authorities having jurisdiction.

        - Compatibility: Provide waterproofing system materials that are compatible with one another and with adjacent materials under conditions of service and application required, as demonstrated by waterproofing manufacturer based on testing and field experience.

      - WATERPROOFING MEMBRANE SYSTEM

        - Cold Fluid-Applied Dual Waterproofing System.

          - Basis of Design Product: Tremco, Inc., TREMproof Platinum Waterproofing System

            - Waterproofing membrane

            - Accessory Materials

            - Protective Waterproofing Course

            - Drainage

2.4		WATERPROOFING MEMBRANE

          - Waterproofing Membrane:

            - Basis of Design Product: Tremco, Inc., TREMproof 250GC

            - VOC Content:  Less than 100 g/L, roller and self-leveling grades.

            - VOC Content:  Less than 160 g/L, trowel detailing grade.

            - Hardness, ASTM D 2240: 80 - 90.

            - Low Temperature Flexibility and Crack Bridging, ASTM C 1305: Pass.

            - Adhesion in Peel, ASTM C 794: 26 lbf/in. (4553 N/m).

      - ACCESSORY MATERIALS

        - General: Accessory materials as described in manufacturer's written installation instructions, recommended to produce complete waterproofing system meeting performance requirements, and compatible with waterproofing material and adjacent materials.

        - Substrate Patching Material: Waterproofing manufacturer's standard trowel-grade filler material.

          - Basis of Design Product: Tremco, Inc., TREMproof 250GC T

        - Primer:  Tremco approved primer meeting VOC limitations and recommended for substrate by waterproofing manufacturer.

          - Basis of Design Product: Tremco, Inc., Vulkem 191 QD.

        - Elastomeric Detail Sheet: Blended thermoset elastomeric sheet reinforced with polyester woven scrim.

          - Basis of Design Product: Tremco, Inc., TREMproof TRA Sheeting.

        - Metal Termination Bars: Waterproofing manufacturer's standard aluminum or stainless steel termination bar, with stainless steel fasteners.

        - Joint Sealant: ASTM C 920, single-component urethane, approved by waterproofing  manufacturer for adhesion and compatibility with waterproofing and accessories.

          - Basis of Design Product: Tremco, Inc., Dymonic 100.

      - PROTECTION COURSE

        - Protective Waterproofing Course: [Waterproofing manufacturer's standard protection course material recommended for application.] [Provide the following:]

          - Basis of Design Product: Tremco, Inc., TREMproof TRA Sheeting.

        - Protection Course: [Waterproofing manufacturer's standard protection course material recommended for application.] [Provide the following:]

          - For vertical applications:

            - Basis of Design Product: Tremco, Inc., TREMproof TRA Sheeting

          - For horizontal slab applications:

            - Basis of Design Product: Tremco, Inc., Tremco 2450 Protection Board, or 6 mil polyethylene

      - DRAINAGE PANELS

        - Drainage Mat: Composite mat with drainage core, filter fabric, and protective polymeric film[, recommended by waterproofing manufacturer for application.] [. Provide the following:]

          - Basis of Desing Product: Tremco, Inc., TREMDrain Series Drainage Mats.

- EXECUTION

      - EXAMINATION

        - Surface Condition:  Before applying waterproofing materials, examine substrate and conditions to ensure substrates are fully cured, smooth, clean, dry, and free from high spots, depressions, loose and foreign particles and other deterrents to adhesion, and conditions comply with manufacturer's written recommendations.

          - Verify concrete and masonry surfaces are visibly dry, have cured for time period recommended by waterproofing manufacturer, and are free from release agents, curing agents, laitance, and other contaminates. Test for waterproofing adhesion per manufacturer's recommended method. Notify Architect of unsatisfactory conditions.

          - Test for capillary moisture per manufacturer’s recommended method.

          - Verify masonry joints are filled with mortar and struck flush.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INTERFACE WITH OTHER WORK

        - Sequencing of Work:  Coordinate sequencing of waterproofing work with work of other sections that form portions of building envelope moisture control to ensure that flashings and transition materials can be properly installed and inspected.

        - Subsequent Work:  Coordinate waterproofing work with work of other sections installed subsequent to waterproofing to ensure complete inspection of installed waterproofing and sealing of waterproofing penetrations necessitated by subsequent work.

      - PREPARATION

        - Clean, prepare, and treat substrates in accordance with waterproofing manufacturer's written instructions.

          - Mask adjacent finished surfaces.

          - Remove contaminants and film-forming coatings from substrates.

          - Remove projections and excess materials and fill voids with substrate patching material.

          - Prepare and treat joints and cracks in substrate per ASTM D 4258 and waterproofing manufacturer's written instructions.

        - Detail Preparation: Prepare non-moving shrinkage cracks, large cracks, construction joints, expansion joints, projections and protrusions, penetrations, drains, and changes in plane in accordance with waterproofing manufacturer's written instructions and details, using accessory materials specified.

          - Adhere strips of elastomeric sheet to moving joints and large cracks by embedding in a layer of cold fluid-applied waterproofing and overlay with coat of cold fluid-applied waterproofing.

        - Transitions to Adjacent Materials: Install elastomeric and composite reinforced flashing to form connect and seal waterproofing material to adjacent components of building waterproofing system, including, but not limited to, roofing system waterproofing, exterior fenestration systems, door framing, and other openings

          - Seal top of through-wall flashings to waterproofing with continuous transition strips of type recommended by waterproofing manufacturer for application.

          - Install elastomeric sheets at terminations of waterproofing membrane according to manufacturer's written instructions.

          - Install termination bars and mechanically fasten to top of elastomeric flashing sheet at terminations and perimeter of waterproofing.

      - WATERPROOFING INSTALLATION

        - General: Apply dual waterproofing system with detailing sealants, waterproofing membrane, primer, protective waterproofing course and drainage according to manufacturer's written instructions. Apply waterproofing material within manufacturer's recommended application temperature ranges.

        - Start application with manufacturer's authorized representative present.

        - Cold Fluid-Applied Platinum WaterproofingSystem, for installation specifics please refer to the TREMproof Platinum Waterproofing System Application Instructions.

          - Apply TREMproof 250GC using roller or squeegee as a single lift of 90 mils and allow to cure.

          - Prime all existing TREMproof 250GC with Vulkem 191QD Primer Allow to dry to a tacky non-transferrable flim.

          - Install an additional 30 wet mil lift of TREMproof 250GC membrane.

          - Embed TREMproof TRA Sheeting into the wet membrane immediately.

        - Terminations: Install terminations of waterproofing membrane in accordance with ASTM C 898 and ASTM C 1471, as applicable to application, at not less than minimum height recommended by waterproofing manufacturer. Overlap waterproofing on to intersecting construction a minimum of 24 inches (600 mm).

        - Correct deficiencies in or remove waterproofing that does not comply with requirements; repair substrates and reapply waterproofing components.

      - PROTECTION INSTALLATION

        - Protection Course: Cover waterproofing with protection course following the written instructions prior to backfilling or subjecting installation to traffic. Overlap protection course joints.

        - Drainage Panel: Place and secure drainage panels using methods that do not penetrate waterproofing. Face geotextile away from deck substrate. Lap edges and ends of geotextile.

      - FIELD QUALITY CONTROL

        - Contractor's Inspector:  Contractor shall engage manufacturer's qualified Inspector full-time during the Work to perform tests and inspections, including documenting of waterproofing prior to concealment.

          - Contractor's Inspector shall measure membrane thickness with pin tester at least once for every 100 sq. ft. (10 sq. m).

          - Provide written report of tests and inspections.

        - Testing Agency: Owner will engage a qualified testing agency to perform tests and inspections.

Retain "Testing Agency" Paragraph below if applicable to Project, and edit to identify party retaining independent agency to perform tests and inspections.

        - Testing Agency: [Owner will engage] [Engage] a qualified testing agency to inspect substrate conditions, surface preparation, waterproofing application, protection, and drainage components, and to furnish reports to Architect.

          - Testing includes EFVM inspection prior to concealing deck waterproof membrane as specified in Section 07 72 73 "Membrane Leak Detection System."

        - Coordination of Inspection: Cooperate with testing agency.  Allow access to work areas and staging.  Notify testing agency in writing of schedule for Work of this Section to allow sufficient time for testing and inspection.

          - Do not cover Work until testing and inspection is completed and accepted.

        - Reporting:  Forward written inspection reports to the Architect within 10 working days of the inspection and test being performed.

        - Correction of Work:  Correct deficient applications not passing tests and inspections, make necessary repairs, and retest as required to demonstrate compliance with requirements.

      - CLEANING AND PROTECTING

        - Clean spills, stains, and overspray resulting application utilizing cleaning agents recommended by manufacturers of affected construction.  Remove masking materials.

        - Protect waterproofing from damage from subsequent work.  Protect waterproofing materials from exposure to UV light for period in excess of that acceptable to waterproofing manufacturer; replace overexposed materials and retest.

END OF SECTION